** USE THIS WHEN SPECIFIED IN MATERIALS DESIGN REPORT **

** USE WHEN MILLED SURFACE WILL BE THE PERMANENT DRIVING SURFACE OR ONLY A THIN LIFT SURFACE WILL BE APPLIED**

# (202MICROMIL, 09/19/24)

# ITEM 2020075 - REMOVE BITUMINOUS PAVEMENT (MICRO-MILLING):

Description:

The work under this item shall consist of micro-milling the existing asphaltic concrete friction course or thin lift pavement treatment to remove surface irregularities, restore grade and/or transverse slope of pavement and provide a texture suitable for a new thin lift surface application, or for use as the final driving surface. The work shall be performed at the locations and depths shown on the plans, as directed by the Engineer, and in accordance with the requirements of these specifications.

# Construction Requirements:

The micro-milling equipment shall be specifically designed to remove material to a controlled line and grade. Milling machines shall be equipped with a drum specifically designed and constructed for micro-milling, consisting of cutting teeth spaced no greater than 1/4 inch apart on center. The configuration of the teeth shall be such that the deviation in elevation between any 2 teeth does not exceed 1/16 inch .The equipment used shall be capable of removing the existing asphaltic concrete friction course uniformly throughout the milled area at the required cross-slope and within 1/10 inch of the specified removal depth. The micro-milling shall be accomplished in a manner which does not destroy the integrity of any underlying pavement surface.

If during milling operation, it is determined that the required average micro-milling depth is greater than the specified micro-milling depth, the contractor shall immediately notify the Engineer and document the situation. Upon approval of the Engineer, the contractor shall proceed with the micro-milling operation. Payment for the removal of additional material will be in accordance with the requirements of Subsection 109.04 of the specifications.

The micro-milled surface shall meet longitudinal profile and cross-slope requirements as shown on plans to the extent achievable within the specified micro-mill depth. The surface finish shall not vary longitudinally or transversely more than 1/8 inch as measured with a 10 foot straightedge. The micro-milled surface shall have a uniform transverse texture pattern of 0.2 inch center to center of each strike area. The target difference between the ridge and valley measurement of the mat surface shall not exceed 1/16 inch. The maximum mean macrotexture depth shall be 0.064 inches, as determined in accordance with Arizona Test Method 742.

At the start of the micro-milling operation, the contractor shall micro-mill a 1,000-foot test section. The micro-milled surface of the test section shall be evaluated by the Engineer for compliance with cross slope, profile smoothness, and maximum mean macrotexture depth requirements. Measurements for compliance shall be taken at a minimum of two random locations, not less than 500 feet apart. If the micro-milled surface is in compliance with the requirements, the contractor can resume the micro-milling operation. If the micro-milled surface is not in compliance, the contractor shall make adjustments to the micro-milling operation and then micro-mill another 1,000-foot test section.

During micro-milling operation, the cross slope, profile smoothness, and maximum mean macrotexture depth shall be determined at a minimum frequency of one test per one-half mile per lane. If, at any time, during the micro-milling operation the Engineer determines that the requirements are not being achieved, the contractor shall stop micro-milling. The micro-milling operation shall not resume until the Engineer is satisfied that the requirements can be met or until successful completion of another test section. The forward speed of the micro-milling machine during milling operation shall not exceed the speed used for the test section. The forward speed of the micro-milling machine shall be checked throughout each production day, or at the discretion of the Engineer.

Micro-milled lanes shall be opened to traffic before the end of each day's work if required by the engineer.  Prior to opening, the contractor shall remove dust, residue, and loose milled material from the micro-milled surface in accordance with the requirements of the plans and specifications, and take appropriate measures to ensure that micro-milled surfaces do not trap or hold water. All required pavement markings removed by the micro-milling shall be restored or otherwise addressed with approved traffic control devices before the roadway is opened to traffic.

In the event of circumstances beyond the control of the contractor, such as equipment breakdown, and the contractor is unable to comply with the requirements in the preceding paragraph, the contractor shall provide and maintain such traffic control devices that the Engineer deems necessary under the circumstances in order to provide safe and efficient passage through the work zone.

Pavement, to be removed by micro-milling, adjacent to manholes, valve boxes, small radius curbs and other fixed objects that produce confined areas shall be removed with milling equipment specifically designed to operate in restricted areas and capable of removing asphaltic concrete friction course of the specified thickness without damage or displacement of the adjacent object. At the discretion of the Engineer, such areas may be excluded from macrotexture testing.

If the milled material is not used on the project, it shall become the property of the contractor.

# Method of Measurement:

Remove Bituminous Pavement (Micro-Milling) will be measured by the square yard.

# Basis of Payment:

The accepted quantities of Remove Bituminous Pavement (Micro-Milling), measured as provided above, will be paid for at the contract unit price per square yard, which price shall be full compensation for the work, complete in place, as described and specified herein. No measurement or direct payment will be made for loading, hauling and disposing of the milled material.