*                          USE STORED SPECIFICATION 109PAVSM WHEN AN                       *

*           INCENTIVE/DISINCENTIVE PAYMENT FOR ASPHALTIC CONCRETE             *

*                            PAVEMENT SMOOTHNESS IS SPECIFIED IN THE                           *

*                                       MATERIALS DESIGN REPORT.                                               *

*	       USE IN CONJUNCTION WITH STORED SPECIFICATION 1001MATL          *

*	FILL IN THE FOLLOWING (SHOWN IN RED TEXT):					*

*		414-3			SPREAD RATE	  (1 place) *

*						PERCENTAGE (ASPHALT-RUBBER)	  (1 place) *

*						PERCENTAGE (LEVELING)	  (1 place) *

*						THICKNESS	  (1 place) *

*		414-3.03			TYPE OF ASPHALT-RUBBER	  (1 place) *

*	USE IN CONJUNCTION WITH THE FOLLOWING: 			 *

*		BID ITEM:							9240170			 *

*		STORED SPECIFICATION:				924CQC			 *

(414ACFAR, 07/18/24)

SECTION 414	ASPHALTIC CONCRETE FRICTION COURSE (ASPHALT-RUBBER):

414-3			Materials:  of the Standard Specifications is modified to add:

For comparative purposes, quantities shown in the bidding schedule have been calculated based on the following data:

| Spread Rate (lb./sq. yd.) | XXX |  |
| --- | --- | --- |
| Asphalt-Rubber, % | XX.X |  |
| Mineral Admixture, % | 1.0 |  |

The estimated target spread rate specified above includes XX percent for leveling to provide a minimum XX-inch thickness above the leveling thickness.

414-3.03		Bituminous Material:  the first paragraph of the Standard Specifications is revised to read:

Bituminous material shall be asphalt-rubber conforming to the requirements of Section 1009 of the specifications.  The asphalt-rubber shall be CRA Type XXXXX.  The crumb rubber gradation shall conform to the requirements of Section 1009 of the specifications.

414-7.04(A)		General Requirements:  the second paragraph of the Standard Specifications is revised to read:

All wheels and tires of compactors shall be wetted with water, or if necessary soapy water, or a release agent in order to prevent the sticking of asphaltic concrete. All other equipment surfaces shall be treated when necessary with a release agent. Only release agents evaluated through AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) are acceptable for use. The results from AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) testing, when tested in accordance with AASHTO T 383, shall conform to the requirements shown in the table below.

414-7.04(A)		General Requirements:  the second row of the “Release Agent Test – Requirement” table of the Standard Specifications is hereby deleted: