** USE ON ALL PROJECTS INVOLVING BLASTING **

** DEFAULT INFORMATION IS SHOWN IN RED TEXT CONSULT WITH GEOTECH AND PROJECT TEAM TO VERIFY THIS INFORMATION IS APPLICABLE FOR THE SPECIFIC PROJECT, MODIFY OR REMOVE AS NEEDED, AND CHANGE TO BLACK TEXT. **

(203BLAST, 06/19/25)

SECTION 203   	EARTHWORK:

203-3.03            Construction Requirements:

(C)         Blasting: of the Standard Specifications is revised to read:

- General:

The contractor shall perform blasting operations, including the storage, handling, and use of explosives and blasting agents, in accordance with the applicable provisions of the Standard Specifications, and all other pertinent Federal, State, and local regulations. Whenever explosives are used, they shall be of such character and in such amount as is permitted by the State and local Laws and ordinances and all respective agencies having jurisdiction over explosives. The contractor is responsible for the effects, including damages, of the contractor’s blasting operations on adjacent public or private property, including objects, structures, and Utilities.

The contractor shall scale all slopes for stability, regardless of excavation technique or slope finish required. Scaling of new or modified rock cuts to remove loosened and unstable material that might become dislodged shall be performed for each cut slope. Rocks or edges protruding from the exposed cut face that could affect the trajectory of falling rock by creating a launching point for materials falling from above onto a roadway shall be removed. Scaling can be achieved by either hand or mechanical means. The adequacy of rock scaling shall be determined by the Engineer.

The contractor shall minimize blast damage behind the trim line. Blast damage includes, but is not limited to widening and loosening of the existing joints, bedding planes, or foliation of the rock mass to remain, displacement of blocks of intact rock to remain, and creation of new fractures on the slope to remain.

Detonators (blasting caps) shall be of an electronic variety. Non-electronic detonators shall not be permitted unless approved by the Engineer for instances including, but not limited to, small shots by volume or hole count/footage as recommended by the blasting consultant.

** MODIFY OR REMOVE RED TEXT AS NEEDED, AND CHANGE TO BLACK TEXT. **

- Road Closures and Safety:

A road closure shall be required for all blasting operations within 1,500 feet of traffic or as otherwise determined by the Engineer. The blast distance radius may be reduced through the use of overburden placement and blast mats for instances where the blasting proximity is close to residential or commercial structures as approved by the Engineer.

Full closures for blasting shall not exceed four hours. Blasting operations shall be performed on Monday through Friday between the hours of 8:00 a.m. to 5:00 p.m. unless otherwise specified in the Special Provisions. Variations to this schedule are subject to the approval of the Engineer. Sufficient equipment shall be on site to quickly clear and reopen at least one lane immediately after the blast, or as otherwise directed by the Engineer. The closure period will begin when the contractor closes the roadway to traffic and will be complete when traffic in each direction begins to flow. The Engineer will document the time of the beginning and end of the closure. The roadway shall not be closed again until all queued traffic has cleared.

If the roadway is not open after a four hour closure, liquidated damages will be assessed in accordance with Subsection 104.04 of the specifications.

The contractor shall observe the entire blast area for a minimum of five minutes following a blast to guard against rock fall before commencing work in the cut.

The Engineer may prohibit or halt the contractor's blasting operations if the required slopes are not being maintained in a stable condition, or the safety and convenience of the traveling public are being jeopardized.

Clearing, scaling, or blasting operations taking place above the traveled roadway shall incorporate berms or other containment measures to control rock migration and protect the public and workers. Scaling of rock slopes adjacent to or above the roadway shall employ devices to contain and collect material for safe removal. These measures shall be implemented at no cost to the Department.

No blasting will be permitted except when the road is closed to public travel. Before blasting takes place, the blasting site and adjoining areas shall be cleared of workers and members of the public.

When night blasting is allowed, the night time frame is defined as the hours starting one-half hour after sunset until one-half hour before sunrise. For purposes of this restriction, “blasting” includes initiating explosives, clearing, and checking blast areas after blasting prior to sounding the all-clear.  Special exceptions may be granted by the Engineer to mitigate misfires.  If night blasting and/or loading is allowed, the entire blast area shall be illuminated from two sides so as to reduce shadows and allow clear observation of the blast area at all times. Drilling and loading (as in handling explosives) can all be completed anytime, including nighttime.

Safety of blasting and related operations is solely the responsibility of the contractor.

No blasting or pioneering shall be performed until pre-scaling and temporary support, if required, is in place.

(3)	Road Maintenance:

The contractor shall patch any pavement used for travel by the public that becomes damaged as a result of blasting or mucking operations. Repair shall be completed as early as practicable after the activity that caused the damage, so as to facilitate the safe and smooth passage of traffic. The Engineer may direct that further blasting be suspended until pavement is satisfactorily patched. The contractor shall have sufficient cold patch asphaltic concrete (AC) material available to repair damaged pavement. Pavement damage resulting in potentially hazardous driving conditions shall be repaired immediately.

Any damage as a result of the blasting operations shall be repaired by the contractor at no additional cost to the Department.

(4)	Blasting Proposal:

The contractor shall submit a blasting proposal for the Engineer's review and approval. The blasting proposal shall be submitted at least two weeks prior to the planned commencement of rock excavation activities. The plan shall be approved before any explosives are delivered to the site. At a minimum, the blasting proposal shall include the following information:

- The proposed location, excavation plan, and description of the test blasting program.

- The proposed sequence of excavation of the various major elements of the project.

- Description of equipment for scaling, drilling, blasting, pioneering, and excavating.

- Method of blasting, excavating, and scaling to form the desired cut slope contour including production blasting and slope finishing in accordance with the visual attainment criteria established for the project, with special note of mechanical techniques to be employed. Station limits where mass excavation will be done by mechanical equipment as opposed to blasting.

- Blasting sequence, timing, dimensions, cleanup approach, and burden adjustments that will meet the road closure limitations specified.

- Intended measures for limiting dust and controlling fly rock that may threaten any structures, utilities, the remaining existing roadway, as well the public using the area, during excavation operations.

- An approved proposal shall be required before any blasting operations, including the test blasts, may commence. The Engineer will either approve the Proposal, or return it for revisions, within two weeks of receipt.

- Names and experience of Blasting Supervisors and Blasters in Charge. Qualifications of all supervisory and technical personnel to be involved in blasting operations. The Blasting Supervisor shall have a minimum of three years experience and completed three projects as a blasting supervisor. Any change in, or addition to, the personnel shown in the approved Proposal must be submitted to the Engineer for approval.

- Methods for and locations of explosives storage, delivery, and handling, a scaled drawing of the location of any magazine to be located within five miles of the Site, and name and contact information for contact person responsible for assuring the security of blasting materials and supplies stored for use on the Project.

- Name, address, and telephone number of all explosives suppliers, and identification of all explosives delivery vehicles and operators.

- Manufacturers' safety data sheets and product data sheets for all explosives, primers, and initiators to be employed.

- Methods Contractor shall employ for traffic control and other public safety precautions in the use, storage, and transportation of explosives.

- Equipment Contractor intends to use in or in support of blasting operations.

- Method of vibration control, vibration monitoring instrumentation, and the identity of the person or persons collecting and analyzing the data.

- Proposed methods to protect the public during blasting operations, including notification to the public through various media, locations and types of signage, fencing, and look-outs.

** MODIFY RED TEXT AND CHANGE TO BLACK TEXT **

(5)	Pre-Blast Condition Survey:

The contractor shall perform a pre-blast condition survey of nearby utilities, structures, and buildings that may be at risk from blasting damage. The specific utilities, structures, and buildings surveyed, as well as the survey method used, shall be acceptable to the contractor's insurance company. The contractor shall be responsible for any damage resulting from blasting. The pre-blast survey records shall be made available to the Engineer for review. Occupants of local buildings within 0.5 mile shall be notified by the contractor prior to the commencement of blasting.

** MODIFY RED TEXT AND CHANGE TO BLACK TEXT **

(6)	Protection of Utilities:

Contractor shall comply with the requirements of nearby utility companies relative to protection of their individual utilities from the effects of blasting operations. The contractor shall also comply with the following requirements when blasting operations are within 1,000 feet of overhead power transmission line areas:

- The contractor shall provide written notification to relevant utility companies a minimum of ten business days prior to blasting within 1,000 feet of any transmission line.

- Once blasting operations have begun, the contractor shall proceed as continuously as practicable with blasting operations in that area.

- The contractor shall prevent fly rock when any portion of any blast is within 300 feet of the outside phase of the closest transmission line. Fly rock prevention measures include covering the entire shot with blasting mats or soil.

* MODIFY RED TEXT AND CHANGE TO BLACK TEXT **

(7)	Control of Vibrations and Air-Blast:

The contractor shall locate seismographs between the blast area and the closest susceptible object, structure, or utility. The contractor shall use seismographs whenever any part of a blast comes within 500 feet of an existing building, box culvert, retaining wall, bridge structure, pipeline, utility pole, transmission tower, or existing landslide.  The Engineer may require the use of seismographs to monitor ground vibrations and/or air blast overpressure at greater distances.

The contractor shall protect all existing facilities from damage from blasting vibrations and air-blast. The contractor shall deploy and monitor an air-blast monitoring system between the main blasting area and the location(s) subject to blast damage or annoyance.

Peak particle velocity of any component of motion (radial/longitudinal, transverse, vertical) shall not exceed the safe limits of any structure subject to vibration damage.

** IF PEAK PARTICLE VELOCITIES ARE TO BE MONITORED, USE THE TABLE BELOW.  MODIFY MAXIMUM VELOCITY VALUES AS NEEDED AND CHANGE TO BLACK TEXT. DELETE THE TABLE IF MONITORING PEAK PARTICLE VELOCITIES WILL NOT BE REQUIRED **

Peak particle velocity of each component shall not be allowed to exceed the safe limits of any structure subject to vibration damage or the following maximum values, whichever is less:

| Structure | Maximum Peak Particle Velocity (in/sec) |
| --- | --- |
| Structures | 2.0 |
| Wells and Aquifers | 2.0 |
| Concrete less than 7 days old | 1.0 |
| Concrete less than 24 hours old | 0.1 |
| Power Poles and Transmission Lines | 2.0 |

The values in the table above may be adjusted based on project conditions during blasting operations as approved by the Engineer.

(8)	Blast Monitoring Plan:

The contractor shall prepare a Blast Monitoring Plan that includes the following elements:

- Types of instruments the contractor proposes for use, including seismographs and transducers for ground vibration, and sensors for air-blast.

- Seismographs must be capable of recording ground motion particle velocity for three mutually perpendicular components of vibration in the frequency range generally found with controlled blasting.

- Air-blast sensors must be specifically manufactured for the purpose of making blasting noise and sound pressure measurements.

- Planned locations (distance and direction) of the monitoring instruments relative to blast locations.

- Proposed methods of adjusting blast hole patterns, detonation systems, and/or stemming to prevent venting of blasts and to bring air-blast and noise levels produced by blasting operations within applicable limits.

- Proposed method(s) of documenting occurrence of fly rock.

- Qualifications and experience of the instrument operators.

The Contractor shall submit a Blast Monitoring Plan to ADOT for review and comment not later than 15 Business Days prior to the first test blast.

* MODIFY RED TEXT AND CHANGE TO BLACK TEXT **

(9)	Test Blasting:

The contractor shall perform a minimum of one test blast at each cut location where blasting is proposed, to demonstrate the adequacy of the proposed Blast Monitoring Plan and the effectiveness of the proposed fly rock control measures. Test blasts shall be a maximum of 5,000 square feet in area and shall be representative of the techniques the contractor proposes for use in full-scale blasting unless otherwise approved by the Engineer due to restrictions related to safety, ground conditions, access, or geography, or any other condition deemed restrictive to the aforementioned. Test Blasts shall meet the requirements for limiting fly rock, road closure restrictions, vibrations and air overpressures. The contractor shall prepare a Test Blast Report for each test blast. Each Test Blast Report must include the following:

- Details of the test blast.

- Locations and details of blast monitoring.

- Fly rock control measures.

- Results of ground vibration and air-blast monitoring.

- Video of the test blast.

- Documentation of fly rock, including particle sizes and travel distances.

- The contractor’s proposed fly rock control measures based on the test blast results.

Not later than five working days after completion of each test blast, the contractor shall submit the Test Blast Report to ADOT for review and comment.

(10)	 Blasting Plan:

Not later than ten working days prior to commencing drilling and blasting operations, the contractor shall submit a Blasting Plan to the Engineer for review and approval. Detailed blasting plans shall be submitted to the Engineer no less than three working days prior to drilling for that individual shot layout by location and design. The Engineer shall approve the detailed blasting plan before any drilling or blasting will be permitted in the area covered by the plan. The Engineer will either approve, or return for revisions, the contractor's detailed blasting plan within two working days of receipt.

The contractor shall prepare a Blasting Plan that includes the following:

- Proposed plan for each shot (to be determined by the Engineer).

(b)	Station limits of each proposed shot.

(c)	Proposed excavation sequence for each cut for each shot.

(d)	Elevations of the tops and bottoms of each lift.

(e)	For each shot, scale drawings showing plan and section views of all variations of the proposed drill pattern, including clearing limits, free face, burden, blast hole spacing, drill-hole location, sub-drill depths, lift height, blast hole diameters, and blast hole angles. The contractor shall account for location and attitude of significant fracturing, rock type changes, faulting, and special circumstances in the shot design.

(f)	For each shot, a loading diagram showing powder factor, type and amount of explosives, primers, initiators, and locations and heights of stemming for all substantial variations within the pattern.

(g)	For each shot, the initiation method and sequence of blast holes, including delay times and delay system.

(h)	Fly rock control measures for each shot.

(i)	Estimated quantities of volume of rock in-place and length of both production and controlled blast drill-holes.

(j)	Vibration criteria, and predicted ground motions at sensors.

(k)	Seismograph placement.

** IF GEO-HAZARD AREAS HAVE BEEN IDENTIFIED, CHANGE THE SENTENCE BELOW TO BLACK TEXT. DELETE THE SENTENCE IF NO GEO-HAZARD AREAS ARE IDENTIFIED. **

(l)	Sensor locations placed in geo-hazard areas as identified by the Engineer.

The contractor shall record each blast on videotape. At the end of each month, the contractor shall submit the unedited videotape recordings to ADOT.

(11) Blasting Report:

The contractor shall prepare a Blasting Report for each blast performed, for whatever purpose including secondary blasting, that includes the following:

(a)	The start and finish of drilling and loading, along with a log of actual explosive loading and any changes in pattern.

(b)	A copy of the blasting shop drawing.

(c)	Approximate average drilling rate, soft seams or faults, and any occurrences of water, lost circulation, voids, stuck drill steel, or other complications to drilling.

(d)	Depth measurements of all production and control holes.

(e)	Name of blasting supervisor and date and time of blast.

(f)	Vibration and air blast records (original printout).

(g)	Include the site-specific ground vibration and air overpressure (air blast) attenuation analyses developed from the blast monitoring data, incorporating the results of Test Blasts and production blasts.

(h)	An attenuation analyses, updated to reflect ongoing project blasting work.

(i)	Multiple attenuation analyses as needed to represent sites with variable geology.

(j)	High-resolution digital video recording of each blast.

Within five working days after each blasting, the contractor shall submit a Blasting Report to the Engineer for review and approval.

** IF AESTHETIC TREATMENTS ARE REQUIRED, MODIFY THE PARAMETERS AS NEEDED AND CHANGE THE TEXT TO BLACK. DELETE THE TEXT IF AESTHETIC TREATMENTS ARE NOT REQUIRED. **

(12)	Aesthetics:

In areas where the project crosses land with special scenic values as determined by the administrating agency (e.g., USFS, BLM, ASL, etc.), the contractor shall prevent or remove deleterious drill-hole traces, machine scars, and marks from machine scaling or other excavation equipment in the final roadway cut faces. Deleterious conditions shall be as identified by the Engineer, and include items listed below:

- Individual drill-holes whose remaining traces total more than 3 feet aggregate length.

- Any portion of any roadway cut bearing drill-hole traces whose aggregate length totals more than 25 percent of the total length of controlled blast holes drilled to form that portion of the cut.

- Machine scars traceable for more than 12 feet which parallel the natural geologic structure, bedding, or principal fracture direction.

- Machine scars that are traceable for more than 6 feet which do not parallel the natural geologic structure, bedding or principal fracture direction.

- Machine scars that are approximately parallel and repetitive (groups of two or more scars).