**     USE ON PROJECTS INVOLVING EARTHWORK

WITH CUTS DEEPER THAN 3 FEET, OR FOR ANY AREAS

IN NON-ATTAINMENT FOR PM-10.  **

**VERIFY USE WITH THE DISTRICT**

**USE THIS STORED SPECIFICATION ON APPLICABLE PROJECTS WITH PAY ITEM 2090005 FURNISH WATER**

**USE IN CONJUCTION WITH STORED SPEC 209WTR **

(203PRWTR, 10/16/03)

SECTION 203	EARTHWORK:

203-2			General:  of the Standard Specifications is modified to add:

Material to be excavated shall be pre-wetted prior to removal.  The Engineer will specify the areas to be pre-wetted.  The contractor shall rip or scarify these areas prior to the application of water, and provide a method to verify penetration of moisture for the full depth of excavation.  The contractor shall provide a sprinkler system for distribution, and apply water at a rate to allow sufficient penetration without excess runoff.

No separate payment will be made for application of the water, or for water trucks, hoses, fittings, sprinklers, meters, and any other equipment required for pre-wetting, or for the labor involved, the cost being considered as included in contract items.  Water for pre-wetting will be paid for in accordance with Section 209.