*                          USE STORED SPECIFICATION 109PAVSM WHEN AN                  *

*           INCENTIVE/DISINCENTIVE PAYMENT FOR ASPHALTIC CONCRETE                *

*                            PAVEMENT SMOOTHNESS IS SPECIFIED IN THE                              *

*                                       MATERIALS DESIGN REPORT                                                   *

*		USE IN CONJUNCTION WITH STORED SPECIFICATION 1001MATL	   *

*		FILL IN THE FOLLOWING (SHOWN IN RED TEXT):				   *

*			407-3		SPREAD RATE			(1 place)    *

*						PERCENTAGE (LEVELING)	(1 place)    *

*						THICKNESS			(1 place)    *

*			407-3.03		TYPE OF ASPHALT BINDER	(1 place)    *

(407ADMIX, 06/19/25)

SECTION 407	ASPHALTIC CONCRETE FRICTION COURSE:

407-3			Materials:  of the Standard Specifications is modified to add:

For comparative purposes, quantities shown in the bidding schedule have been calculated based on the following data:

| Spread Rate (lb./ sq. yd.) | XXX |  |
| --- | --- | --- |

The estimated target spread rate specified above includes XX percent for leveling to provide a minimum XX inch thickness above the leveling thickness.

407-3.03		Bituminous Material:  the first paragraph of the Standard Specifications is revised to read:

Asphalt cement shall be a performance grade (PG) asphalt binder, conforming to the requirements of Section 1005.  The type of asphalt binder shall be PG XXXX.

407-7.04(A)		General Requirements:  the second paragraph of the Standard Specifications is revised to read:

All wheels and tires of compactors shall be wetted with water, or if necessary soapy water, or a release agent in order to prevent the sticking of asphaltic concrete. All other equipment surfaces shall be treated when necessary with a release agent. Only release agents evaluated through AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) are acceptable for use. The results from AASHTO Product Evaluation & Audit Solutions (formerly NTPEP) testing, when tested in accordance with AASHTO T 383, shall conform to the requirements shown in the table below.

407-7.04(A)		General Requirements:  the second row of the “Release Agent Test – Requirement” table of the Standard Specifications is hereby deleted:

407-8			Method of Measurement:  of the Standard Specifications is revised to read:

Asphaltic concrete will be measured by the ton for the mixture actually used, which will include the weight of mineral aggregate, bituminous material, and mineral admixture.  Measurement will include any weight used in construction of intersections, turnouts, miscellaneous items, or other surfaces.

407-9			Basis of Payment:  of the Standard Specifications is revised to read:

The accepted quantities of asphaltic concrete, measured as provided above, will be paid for at the contract unit price per ton, adjusted if necessary for spread, which price shall be full compensation for the work, complete in place, as specified herein.

If the quantity in a spread lot is found to vary by more than +5.0 percent from the required quantity, as determined in accordance with Subsection 407-6.04 of the specifications, no payment will be made for the material which exceeds the +5.0 percent, including asphalt cement and mineral admixture.

The Engineer may exclude asphaltic concrete from the spread lot if the Engineer determines that the proposed use of the material or the existing surface conditions are not conducive to the use of spread lots.

No separate measurement or payment will be made for asphalt cement, the cost being considered as included in the cost of the asphaltic concrete.

A reduction in payment to the contractor for asphaltic concrete will be made for quantities of asphalt cement that do not meet the requirements of Section 1005 as determined by corresponding test results. Adjustments in payment shall be made in accordance with the requirements of Section 1005 of the specifications.

No separate measurement or payment will be made for mineral admixture, the cost being considered as included in the cost of the asphaltic concrete.

When required in the Special Provisions, payment for smoothness shall be made in accordance with the requirements of Subsection 109.13 of the specifications.

When lime water is used, no separate payment will be made for the lime water or its application, the cost being considered as included in this contract item.