(605STRF, 8/21/25)

SECTION 605	STEEL REINFORCEMENT:

605-3.01		General: the sixth paragraph of the Standard Specifications is revised

to read:

Reinforcement shall be accurately fabricated and placed as shown on the plans and shall be firmly held in place by wire ties at all intersections and splices with 16 gauge or heavier tie wires and with precast mortar blocks or ferrous metal chairs, spacers, metal hangers, supporting wires, or other approved supports at the spacing necessary to maintain the specified clearance of the rei n forcing steel. The use of pebbles, broken stone, concrete masonry blocks, brick, metal pipe, or wood blocks will not be permitted for the purpose of spacing or support.

The contractor has the option to submit a request detailing the use of contract or buried bars, including supporting drawings, for review and approval by the Engineer.

For reinforcement spacing is greater than or equal to 12 inches in either the longitudinal or transverse directions, the reinforcement shall be tied at a minimum every alternate intersection.

Where reinforcement spacing is less than 12 inches in either the longitudinal or transverse directions, the reinforcement shall be tied at a minimum, every third intersection.

The contractor may submit a request detailing an alternative tying plan for review and approval by the Engineer.