** FILL IN INFORMATION (SHOWN IN RED TEXT) AND CHANGE TO BLACK TEXT**

*** INCLUDE ADDITONAL REQUIREMENTS OF THE NATIONAL FOREST SUCH AS CLEARING PLAN, MINING PLAN, EROSION CONTROL AND SEEDING PLAN, IF APPLICABLE ***

## (107FS, 07/20/23)

SECTION 107	LEGAL RELATIONS AND RESPONSIBILITY TO PUBLIC:

## 107.12		Forest Protection: of the Standard Specifications is revised to read:

General:

If a project or a material pit is located on land which is under the jurisdiction of the U.S. Department of Agriculture (USDA) Forest Service, the contractor shall comply with all applicable State and Federal laws, Federal rules and regulations, and the requirements of the National Forest in which the work is located.

Throughout this subsection, the term Forest Service Officer is used.  This person shall be understood to be the District Ranger or any other person authorized or duly appointed to act in all matters affecting the XXXXXXXXXXX National Forest lands, XXXXXXXX Ranger District.

Forest Service Officers on this project are as follows:

|  |  | Phone |  |
| --- | --- | --- | --- |
|  | Name | Day | Night |
| District Ranger | XXXXXXXX | (XXX) XXX-XXXX | (XXX) XXX-XXXX |
| Fire Dispatcher | XXXXXXXX | (XXX) XXX-XXXX | (XXX) XXX-XXXX |

The contractor shall comply with the following minimum requirements for activities within the National Forest:

The contractor shall do everything reasonably within the contractor's power to prevent wildland fires and shall not dispose of material by burning without a written permit from the Forest Service.

The contractor shall not cut timber or brush or commence work in any material pit unless a permit to do so has been obtained from the Forest Service.

The contractor shall fully repair all damage caused by its operations and provide maintenance commensurate with the contractor's use of National Forest roads and trails.

The contractor shall fully comply with all requirements regarding air quality.

Fire Plan:

The contractor shall submit a Fire Plan for approval by the Engineer and the Forest Service Officer prior to any construction activities. The contractor shall prepare the Fire Plan considering the details outlined herein.

The following outlines the mutual responsibilities of the Forest Service, the Department and the contractor to ensure effective prevention and prompt suppression of all fires within the right-of-way and other work areas.

- General:

Fires discovered by the contractor and its employees shall be promptly reported to the Engineer and the Forest Service Officer.  The contractor and its employees shall make reasonable attempts to control and extinguish fire without endangering life of contractor’s personnel or public on any and all fires within the right-of-way and work areas and within a half-mile zone adjacent thereto.  Fires shall be attacked promptly so as to control them while they are small.  Fires shall not be abandoned until the Forest Service Officer declares the fire to be out or relieves the contractor and its employees.

The contractor shall place its equipment and employees at the disposal of the Forest Service Officer for the purpose of fighting fires in accordance with the approved Fire Plan.

Equipment shall be operated by employees of the contractor.  The Forest Service Officer will assume no responsibility toward such equipment for ordinary wear or damage due to negligence, carelessness, or willful misconduct.

- Personnel:

The contractor shall designate at least one representative to train and supervise each construction crew and the individuals designated to perform the duties of fire guard for wildland fire prevention, detection, and suppression.  Each such representative shall be named in the Fire Plan.

At contractor’s discretion, fire guards may be trained to knowledge standards provided by free Federal Emergency Management Agency (FEMA) NWCG S-190 Introduction to Wildland Fire Behavior (fema.gov) and NWCG S-130 Wildland Firefighter Training (fema.gov) or equivalent training.

The contractor shall provide a list of trained fire guards in the Fire Plan, for all activities involving internal combustion engines and where power-driven equipment has been operated during the day.

The fire guards shall constantly perform their duties during the operating hours and for three additional hours after the construction operations ends for the day, when the proposed Fire Precaution Plan is Plan “B” or “C” as specified in Subsection 107.12(B)(14) of the specifications. The additional work hours of the fire guards will be compensated in accordance with the requirements specified in subsection 104.02 of the specifications. The fire guard may perform contract work in conjunction with fire guard duties during operating hours.

Fire guard on one operating area shall satisfy the requirements on adjacent areas if the travel time with available transportation to reach an adjacent area is not in excess of 10 minutes.

Each fire guard shall be physically able, vigilant, and trained to prevent, detect, and report any wildland fires and to promptly and efficiently take suppression action with available required firefighting equipment and personnel on any wildland fire within the project limits. Each fire guard shall be equipped with a vehicle and a fire tool cache consisting of a cache box, two 4-to-5 gallon backpack pumps filled with water, two size 0 shovels in good working condition, two Pulaskis, and two McLeod tools maintained in serviceable condition.

- Reimbursement:

Reimbursement to the contractor for equipment utilized, including operators, will be made by the Forest Service in accordance with rates which have been previously negotiated and agreed upon.  These rates will be established prior to mobilization of personnel and equipment to be used for the suppression of fires.

The Forest Service will not reimburse the contractor, for the suppression of fires started by them.

The contractor, in suppressing fires caused by others, including lightning-caused fires, in work areas or within a half-mile zone of work areas will be reimbursed by the Forest Service for such suppression.

In emergencies, the contractor may be called upon to suppress fires on other National Forest lands outside the normal half-mile zone within the project proximity.  The contractor will be reimbursed by the Forest Service for such suppression.

If the Forest Service requests that any equipment be made available on standby status, reimbursement to the contractor will be made at a rate established by negotiation.

The Department’s publication, Requirements for the Utilization of the Schedule of Equipment Rental Rates for Equipment Used on Force Account Work, will not be applicable when the contractor's equipment is being utilized under the direction of the Forest Service Officer in the suppression of fires.

- Tool Cache:

The contractor shall furnish and maintain at a minimum, one tool cache where tools are clean of rust and foreign material; with sharp cutting edges; in good working condition; and sanded and tightly fitted handles. A tool cache shall contain five long-handled, round point, size No. 0 or larger shovels; five McLeod tools; two double-bit axes; two brushhooks; two backpack sprayers of 3-5 gallon capacity and two Pulaskis with sheaths.  These tools are required separate from, and in addition to, the tools required on equipment and tools under the possession of fire guards.

A proper tool mix shall be specified in the project’s Fire Plan for approval. Fire tools shall be located in the active operating area of the project or as stated in the approved Fire Plan.

The fire tools shall be used only for suppressing wildland fires. All personnel performing the contractor’s operations shall be equipped with at least one approved fire tool.

- Vehicles and Construction Equipment:

All passenger carrying vehicles, including light pickup trucks shall be equipped with one long-handled, round-pointed shovel in good working condition, and one ABC chemical fire extinguisher not less than 2-1/2 pounds capacity.

Each internal combustion fuel carrying truck, loader, skidder, heavy truck, tractor or any other type of equipment, including but not limited to, earth moving equipment, shall be provided with one long-handled round-pointed shovel in good working condition, and one 5-1/2 pound capacity ABC dry chemical fire extinguisher.

Shovels and fire extinguishers shall be so mounted as to be readily reached from the ground.

All internal combustion engines, including those on tractors, jammers or any stationary or mobile equipment, shall be equipped with a spark arrester qualified and rated under USDA Forest Services Standard 5100-1a, “Spark Arrester Guide” or the latest revision of Society of Automotive Engineers (SAE) recommended practice J350, "Spark Arrester Test Procedure for Medium Size Engines” unless it is:

  - Equipped with a turbine-driven exhaust supercharger such as the turbocharger with no exhaust bypass.

  - A multi-position engine which meets the performance levels set forth in SAE recommended practice J335B, "multi-positioned small engine exhaust fire ignition suppression” as now or hereafter amended.

  - A passenger carrying vehicle or light truck, or medium truck up to 40,000 GVW, used on roads and equipped with a factory designed muffler and an exhaust system in good working condition.

  - A heavy duty truck, such as a dump truck, or other vehicles used for hauling, and used only on roads or established accesses and equipped with a factory designed muffler and with a vertical stack exhaust system extending above the cab.

Heavy equipment equipped with an exhaust driven turbocharger in good working order and with no exhaust bypass will qualify.  A straight, mechanically driven supercharger does not qualify.  Light trucks up to two-ton, pickups, jeeps, and passenger cars shall have effective mufflers and exhaust pipes comparable to the manufacturer's standard equipment installation.

All internal combustion engine driven vehicles and equipment will be inspected by the Forest Service Officer prior to initial operation.  All such vehicles and equipment arriving on the work after the initial inspection shall be reported to the Forest Service Officer for inspection prior to operation.  All equipment will be inspected periodically after the initial inspection to ensure that spark arresters or turbocharger are in working order.  All operators shall submit their equipment to the Forest Service Officer for inspection upon request. Vehicles and equipment not approved for use shall be repaired to meet the requirements specified herein, re-inspected, and approved by the Forest Service Officer prior to use.

All 1975 and later model passenger vehicles and pickups equipped with a catalytic converter shall have a sticker clearly displayed on the dashboard warning the driver of the fire hazard of driving the vehicle over or parking the vehicle where tall grass or other flammable materials can come in contact with the catalytic converter.

The contractor shall make daily inspections of all internal combustion engines, stationary and mobile, to ascertain that spark arresters and mufflers are whole and effective, and that there are no connections that can leak burning particles.

All crawler tractors, rubber-tired tractors, and equipment with power source that are suitable for wildland fire suppression work, shall be equipped with two factory type headlights and one backup light, or brackets mounted for portable self-contained battery operated lights. These portable lights shall be furnished and maintained by the contractor at a location approved by the Engineer.

Exhaust equipment described in this Subsection, including spark arresters and mufflers, shall be properly installed and constantly maintained in serviceable condition.

- Smoking:

There shall be no smoking while working.  This requirement applies to those activities which require that work be performed in an uncleared area, such as clearing and fencing operations.  Smoking shall be restricted to areas designated by the Forest Service Officer and such areas shall be a minimum of 2 feet in diameter which have been cleared to mineral soil.  All smoking material shall be completely extinguished before leaving these areas.

There shall be no smoking when driving on National forest lands except when traveling on a graveled, oiled, or otherwise surfaced road.  Smoking material shall be extinguished either in ash trays or on a spot cleared to mineral soil.

All smoking closures or other restrictions which may be put into effect shall be carefully observed.

- Power Saws and Power Equipment:

Power saws shall be equipped with an approved and serviceable spark arrester/muffler.  The spark arrester shall be maintained in effective working order, meeting either Department of Agriculture, Forest Service Standard 5100.1a or appropriate Society of Automotive Engineers (SAE) recommended practice J335 and J350 [36 CFR 261.52(j)], as revised to the date of the opening of the proposals.

Power saws or power equipment shall be filled only in an area which has been cleared to mineral soil.  They shall not be started at the place where they have been filled.  Mufflers shall be kept in place at all times and the spark arrester screens shall be checked daily.  Broken or burned screens shall be promptly replaced. Power saws and power equipment shall be moved at least 10 feet from the place of fueling or refueling before use.

Power saw operators shall check the sawdust at each tree felled before leaving to make sure that no fires have been started.  Each operator shall carry with their power saw; a size No. 1, long-handled, round-pointed shovel in good working condition; a fuel can; and a chemical pressurized fire extinguisher of not less than eight ounce capacity by weight.

- Burning:

No burning of slash, camp refuse, or other debris or any other burning will be permitted without a daily written permit from the Forest Service and a burn permit issued by the ADEQ, Office of Air Quality.

- Fires:

No camp or lunch fires will be permitted except in designated, cleared areas approved by the Forest Service Officer.  Any request for lunch fires, warming fires or campfires at an area except those which have been approved shall be made in writing to the Forest Service Officer.

No fires will be allowed when the Fire Precaution Plan “C” or “D” as specified in the Subsection 107.12(B)(14) of the specifications is in effect within the project limits.

- Campsites, Parking and Storage Areas:

Campsites, parking, and storage areas on Forest lands shall be approved by the Forest Service Officer prior to use.

All campsites shall comply with all applicable Federal, State, County and City statutes and, ordinances concerning safety, health, and sanitation.  Sites shall be cleared of brush, litter, grass or other flammable debris for a radius of 50 feet, maintained, and used in accordance with Forest Service regulations in order to keep the danger of fires to a minimum.  The layout of buildings, tanks, trailers, sanitary facilities, etc., shall be approved by the Forest Service Officer.

All chimneys shall be equipped with 1/2-inch mesh screen for spark arrestors.  Fire extinguishers approved by the Forest Service Officer shall be placed in readily accessible places in campsites and storage areas.

No fires except those as may be approved in writing by the Forest Service Officer will be permitted.  When required by the Forest Service Officer, the contractor shall maintain a cleared fire line around campsites.  Upon cessation of use of the campsite, the contractor shall clean up and restore the site to a condition satisfactory to the Forest Service Officer.  Such restoration shall give consideration to the minimizing of erosion.

- Welding:

Welding shall be done only at sites which have been cleared to mineral soil. An area within 10-foot radius shall be cleared down to mineral soil before the commencement of the welding operations. Immediately following welding and metal cutting operations, and before leaving the site, careful inspections shall be made to detect and extinguish smoldering materials.  A round pointed long handle shovel in good working condition, a 5 or 10-gallon container of water with pump, and a 5 pound fire extinguisher shall be available at each welding site.

A fire guard shall remain on duty during the welding process and at least one additional hour after the process is completed. If the additional work hours fall beyond the normal operating hours of the construction work shift, such additional work hours will be compensated in accordance with the requirements specified in Subsection 104.02 of the specifications. Welding is prohibited on projects when the proposed Fire Precaution Plan is “D” as specified in Subsection 107.12(B)(14) of the specifications.

- Blasting:

Electric caps shall be required; fuse caps are prohibited.  Primer cord shall be understood to be an explosive and not fuse.

- Gas and Oil Storage, and Service Areas:

The location of equipment service areas and gas and oil storage shall be approved in writing by the Engineer. All areas shall be cleared of brush, litter, grass or other flammable debris for a radius of 50 feet.

- Emergency Fire Precautions:

The emergency fire precautions shall be in accordance with the requirements specified in CT7.22 “Emergency Fire Precautions (5/2008)” and as specified herein.

In case of an emergency, the contractor shall restrict the operations by following the Emergency Fire Precaution Schedule specified herein.

Staged restriction levels will be determined by the Forest Service Officer. The process will be a mix of quantitative and subjective measures considering local conditions and issues when deciding to implement fire restrictions and/or area closures.

In areas where emergency fire precautions are to be followed, the contractor shall provide a fire guard.

| EMERGENCY FIRE PRECAUTION SCHEDULE |  |
| --- | --- |
| FIRE RESTRICTION/CLOSURE “STAGE” |  |
| “STAGED” RESTRICTION LEVELS | FIRE PRECAUTION PLAN |
| NO RESTRICTIONS | A |
| STAGE I | B |
| STAGE II | C |
| STAGE III PARTIAL/FOREST CLOSURE ** | C or D |
| STAGE IV (TOTAL FOREST CLOSURE) | D |
| RED FLAG WARNING (Issued by National Weather Service) | D |

The Fire Precaution Plans are described as follows:

| Fire Precaution Plan | Description |
| --- | --- |
| A | The contractor shall comply with all the requirements specified in Subsection 107.12 of the specifications. |
| B | The contractor shall comply with all the requirements specified in Subsection 107.12 of the specifications except designated areas for smoking and warming or cooking fires will require a written permit from the Forest Service. |
| C | - All power tools shall not be used between 9:00 AM and 8:00 PM, except power tools may be used between 9:00 AM and 2:00 PM on areas cleared to mineral soil. - Loading is authorized to continue between 12:00 Noon and 2:00 PM, on areas cleared to mineral soil. - Hauling trucks shall be out of the Construction Area to a surfaced road by 2:00 PM. - All mechanical equipment used for blasting; welding; metal cutting; and clearing and grubbing shall not be used between 12:00 Noon and 8:00 PM. - Operations on mineral soil involving road excavation, watering, grading, surfacing, rock crushing, and/or other equipment maintenance may continue with the approval of the Engineer and the Forest Service Officer. No smoking, warming or cooking will be permitted at any time. |
| D | The contractor shall shutdown all operations; except operations on mineral soil involving road excavation, watering, grading, gravel surfacing, and rock crushing may continue with special Forest Service permit. |

The Forest Service Officer may adjust this Fire Precaution Plan for local weather conditions within the project limits.

The process to notify any change to the Fire Precaution Plan by the Forest Service Officer to the Engineer shall be discussed at the pre-construction conference or partnering meeting. These changes shall be approved by the Engineer and the Forest Service Officer through a written agreement.

The approved change to the Fire Precaution Plan will be communicated to the contractor by the Engineer through phone, email or in-person immediately and prior to the forthcoming work shift.

Projects which are outside the boundaries of the partial forest closure may continue to operate under Industrial Fire Precaution Plan “C” operating criteria as agreed upon between the Forest Service Officer and the Engineer in writing.

Projects within the boundaries of the proclaimed partial forest closure area shall operate under Fire Precaution Plan “D”.