*USE IN CONJUNCTION WITH 1006PCC STORED SPECIFICATION*

(601PCC, 09/21/23)

SECTION 601	CONCRETE STRUCTURES:

601-3.03(C)	Pumping Concrete: of the Standard Specifications is revised to read:

Where concrete is conveyed and placed by mechanically applied pressure, the equipment shall be of suitable type and shall have adequate capacity for the work.  The concrete shall not flow either over or through any piping, fittings or equipment which is fabricated of aluminum or aluminum alloys.  The operation of the pump shall be such that a continuous stream of concrete without air pockets is produced.  Excessive segregation due to high velocity discharge of the concrete will not be permitted.  When pumping is completed, the concrete remaining in the pipeline, if it is to be used, shall be ejected in such a manner that there is no contamination of the concrete or segregation of the ingredients.  Standby equipment shall be readily available to replace initial pumping equipment should breakdown occur.