(733SBRB, 12/21/23)

SECTION 733 	SIGNAL INDICATIONS AND MOUNTING ASSEMBLIES:

733-2.01(A)(4)		Backplates: of the Standard Specifications is revised to read:

Louvered backplates shall be furnished and installed on all vehicular signal sections. Anodized aluminum sheet, 16 gage, shall be used. All 12-inch signal faces shall have 5-inch backplates installed. All 8-inch signal faces shall have 8-inch backplates installed. All backplates shall have a 2-inch fluorescent yellow border of retroreflective tape conforming to ASTM Type XI reflective sheeting requirements.