*	USE ON PROJECTS WITHIN MARICOPA COUNTY		*

(104DUST, 11/01/95)

SECTION 104	SCOPE OF WORK:

104.08		Prevention of Air and Noise Pollution:  of the Standard Specifications is modified to add:

For work performed within Maricopa County, the contractor will be required to prepare a comprehensive fugitive dust control plan, in accordance with the guidelines established in Rule 310 of Maricopa County Regulation III, Control of Air Contaminants.  The contractor may contact Maricopa County, Division of Air Pollution Control, to purchase a copy of the guidelines.  The contractor shall complete and submit the control plan with the permit application, and obtain approval prior to construction or any other activities which may produce dust pollutants.

Some of the measures which the contractor may use to control or minimize fugitive dust include:  increased use of water or chemical dust suppressants, cease work temporarily during high winds, reducing vehicle speeds and number of trips, maintaining freeboard of three inches or more in hauling, and covering or stabilizing stockpiles.  The contractor shall be required to cover haul trucks with tarps or other suitable enclosures.

No separate payment will be made for preparation and implementation of the fugitive dust control plan, the costs being considered as included in the price of contract items.