** USE THIS WHEN SPECIFIED IN MATERIALS DESIGN REPORT **

** USE ON PROJECTS INVOLVING MILLING AC FRICTION COURSE ON PCC**

# (202PCMIL, 10/21/21)

# ITEM 2020078 - REMOVE BITUMINOUS PAVEMENT FROM PCC (MILLING):

Description:

The work under this item shall consist of milling the existing asphaltic concrete from the underlying PCC at the locations and depths shown on the plans, as directed by the Engineer, and in accordance with the requirements of these specifications.

# Construction Requirements:

The milling equipment shall be specifically designed to remove material to a controlled line and grade. The equipment used shall be capable of removing the existing asphaltic concrete uniformly throughout the milled area down to the PCC surface and at the PCC cross-slope. The milling shall be accomplished in a manner which does not destroy the integrity of the underlying PCC surface.

If during milling operation, it is determined that the required average milling depth is greater than the specified milling depth, the contractor shall immediately notify the Engineer and document the situation. Upon approval of the Engineer, the contractor shall proceed with the milling operation. Payment for the removal of additional material will be in accordance with the requirements of Subsection 109.04 of the specifications.

Care shall be taken to ensure that the underlying PCC is not damaged during the milling operation. To avoid damages to the underlying PCC, the contractor shall make adjustments to the milling operation which include but are not limited to operating the milling machine with new teeth/holders, operating at a lower speed, and not allowing the milling cutter head to mill over the PCC construction joints.

Each milling machine shall have a minimum of one ground person per machine to constantly monitor the varying depth of the asphaltic concrete that is being removed. The milling machine operator shall monitor the speed of the milling machine while milling the asphaltic concrete to minimize damage of underlying PCC resulting from the variability in the depth of asphaltic concrete.

In the event of circumstances beyond the control of the contractor, such as equipment breakdown, or if the production of the milling has been stopped by the Engineer and the contractor is unable to finish the designated area, the contractor shall provide and maintain such traffic control devices that the Engineer deems necessary under the circumstances in order to provide safe and efficient passage through the work zone, including cleanup, PCC repairs, and striping.

If the Engineer deems it to be warranted, the Engineer will require that the contractor provide for the surface drainage of areas where the pavement surface has temporarily been removed.

Pavement, to be removed by milling adjacent to manholes, valve boxes, small radius curbs and other fixed objects that produce confined areas shall be removed with milling equipment specifically designed to operate in restricted areas and capable of removing asphaltic concrete of the specified thickness without damage or displacement of the adjacent object.

If the milled material is not used on the project, it shall become the property of the contractor.

# Method of Measurement:

Remove Bituminous Pavement from PCC (Milling) will be measured by the square yard.

# Basis of Payment:

The accepted quantities of Remove Bituminous Pavement from PCC (Milling), measured as provided above, will be paid for at the contract unit price per square yard, which price shall be full compensation for the work, complete in place, as described and specified herein.

No measurement or direct payment will be made for loading, hauling and disposing of the milled material.