*      USE IF THE PROJECT HAS BRIDGE REMOVAL ITEMS (WHOLLY OR IN PART)      *

(202BRGREM, 05/20/21)

SECTION 202	REMOVAL OF STRUCTURES AND OBSTRUCTIONS:

### 202-3.05	Removal of Bridges: of the Standard Specifications is revised to read:

The removal of existing bridges, either wholly or in part, shall be as shown on the project plans or as described in the specifications.  Bridge removal operations shall be conducted in such a manner as to cause the least interference to public traffic.

At least 10 days before beginning bridge removal over or adjacent to public traffic or railroad property, the contractor shall submit to the Engineer details of the removal operations showing the methods and sequence of removal and equipment to be used.

When total bridge removal is specified, all materials designated for salvage, such as structural steel, structural steel members, timber, and other reusable materials shall be carefully dismantled, removed and salvaged in accordance with the requirements of Subsection 202-3.01 of the specifications. Steel members shall be match marked as directed by the Engineer.

Piling, piers, abutments, footings, and pedestals shall be removed to at least 1 foot below ground line or 5 feet below finished subgrade elevation unless specified otherwise in the Special Provisions or on the project plans.

When partial bridge removal is specified or alteration of an existing bridge requires removal of portions of the existing structure, such removal shall be performed with sufficient care as to leave the remaining portion of the structure undamaged.

In case of damage to the existing bridge structure, the contractor shall make all necessary repairs at no additional cost to the Department. Reinforcing steel extending from the remaining portion of the structure shall be protected, cleaned, and incorporated in the new portion of the structure in accordance with the details shown on the project plans or as directed by the Engineer.

Flame cutting, saw cutting, or shear cutting may be used for removing, widening, or modifying bridges provided the contractor complies with all protection, safety and damage requirements. When lead-based paint is present, the contractor shall remove a minimum of 6 inches of the paint on each side of the cutline prior to flame cutting or saw cutting. Prior to shear cutting, a minimum of 36 inches of lead-based paint shall be removed on each side of the cutline, or as determined by the Engineer. Removal and disposal of the lead-based paint shall be performed in accordance with the requirements specified in the Special Provisions.

Explosives shall not be used in bridge removal operations unless approved by the Engineer.

No removal operations shall commence until the Engineer has reviewed and approved the contractor's removal plan. If the Engineer deems the removal equipment to be unsatisfactory, the contractor shall submit a revised removal plan to the Engineer for review and approval.

Before beginning concrete removal operations involving the removal of a portion of a monolithic concrete element, a saw cut approximately 1 inch deep shall be made to a true line along the limits of removal on all faces of the element which will be visible in the completed work.

Concrete shall be disposed of per the requirements specified in Subsection 202-3.03(A) of the specifications.