***USE IF THE TOTAL QUANTITY ESTIMATE FOR ALL WATER

IS MORE THAN 50,000 MGAL***

**	USE STORED SPECIFICATION 203PRWTR IF APPROPRIATE.	**

(209WTR, 02/16/23)

SECTION 207	DUST PALLIATIVE: of the Standard Specifications is hereby deleted.

SECTION 209	FURNISH WATER: is hereby added to the Standard Specifications

209-1			Description:

Except as specified below, the work under this section shall consist of furnishing all water required for construction within the project limits.  This work shall include securing and transporting water to the project site.  All costs, and all labor, equipment, and materials required to secure, transport, and furnish water to the project limits, including, but not limited to, hauling, pumping, and piping water to the project site from canals, rivers, lakes and wells, shall be considered as included in the work.

The work under this section shall also include furnishing all water, as specified above, for the control of dust as considered necessary for public safety and convenience of the traveling public, for the reduction of the dust nuisance to adjacent property, for the allaying of dust in non-commercial crusher and pit operations, both onsite and offsite, and on roads used to haul material, and for other purposes as directed by the Engineer.

When included in the contract or approved by the Engineer, this item shall also include furnishing water to the project site for those pre-wetting areas specified in the special provisions, or as approved by the Engineer.

Water used for materials prepared in an onsite mixing or processing plant, such as, but not limited to concrete, lean concrete base, or soil cement, will not be included in this work.

Water used in landscape establishment, as specified in Section 807 of the specifications, will not be included in this work.

209-2			Blank

209-3			Construction Requirements:

The use of pressure pumps and spray bars on all sprinkling equipment used on the project will be required.  The use of gravity flow spray bars and splash plates will not be permitted for dust control operations.

Water for use in compaction or for pre-wetting shall be applied in accordance with Section 203 of the specifications.

Water applied for dust control shall be as approved or directed by the Engineer.  The contractor shall provide appropriate equipment for effective control of dust.

Water conservation by using alternative dust control treatments is encouraged.  The contractor may propose alternative dust suppressants for subgrades, embankments and other areas within the project, for haul roads, or for controlling dust at equipment yard sites.  When the use of chemical dust suppressants is proposed in lieu of water, the contractor’s submittal shall be in accordance with the value engineering process as specified in Subsection 104.13 of the specifications.

209-4			Method of Measurement:

The work will be measured by the unit of 1,000 U.S. gallons of water (MGAL).  Measurement will be made by means of sealed and certified flowmeters.  Flowmeters shall be isolated and soley utilized for work eligible for payment under this item.

For small quantities, or where a sealed and isolated flowmeter is not feasible, measurements can be made in tanks or tank trucks of certified capacity, or by other means proposed by the contractor, if approved by the Engineer.

209-5			Basis of Payment:

The accepted quantity of water used in construction, and for control of dust and, if required, for pre-wetting, secured and transported to the project site, and measured as provided above, will be paid for at the contract unit price, which price shall included all costs, including all labor, equipment, and materials necessary for furnishing water as required.

No payment will be made for the labor, equipment, and materials required for furnishing water from sources located within the project limits.

The contract unit price for Furnish Water will not include payment for application or distribution of the water within the project limits, or for water trucks, hoses, fittings, sprinklers, meters, and any other equipment required to distribute and apply the water, or for the labor involved.

The cost for application and distribution of water required for construction shall be considered as included in other related contract items, such as earthwork, subgrades, base courses, and backfill materials as appropriate.

The costs for distributing and applying water for pre-wetting, if required, including all fittings and equipment, and the labor involved, will be considered as included in the appropriate contract items for earthwork.

The cost for distributing and applying water for dust control, including the water truck and all fittings and equipment, and the labor involved, will be considered as included in other contract items.

No measurement or payment will be made for water or other suppressants used for prevention of dust from any material sources, pit or crusher operations located outside of the project limits, or for related haul roads, the cost being considered as included in other contract items.  However, should the Department require the use of a specific source for the project, water or other dust suppressants used for prevention of dust at the source and related haul road will be measured and paid for as specified herein.

Except for sources required by the Department, no measurement or payment will be made for water used for pre-wetting off the project site.

No adjustment to the bid price for furnishing water, as specified in Subsection 104.02 of the specifications, will be allowed as a result of a value engineering proposal for alternative dust suppressants.

Payment for chemical dust suppressants, when approved, shall be in accordance with Subsection 104.13 of the specifications.  Other alternative dust suppressants, when approved by the Engineer, will be paid for in accordance with Subsection 109.04 of the specifications.