*                         USE STORED SPECIFICATION 109PAVSM WHEN AN                        *

*            INCENTIVE/DISINCENTIVE PAYMENT FOR ASPHALTIC CONCRETE            *

*                               PAVEMENT SMOOTHNESS IS SPECIFIED IN                                *

*                                        THE MATERIALS DESIGN REPORT                                       *

*	USE IN CONJUNCTION WITH THE FOLLOWING: 					 *

*		BID ITEM:							9240170			 *

*		STORED SPECIFICATIONS:			924CQC			 *

*									        1001MATL			 *

*	FILL IN THE FOLLOWING (SHOWN IN RED TEXT):					 *

*		415-3			UNIT WEIGHT					(1 Place)	 *

*					PERCENTAGE					(1 Place)	 *

*		415-3.04		TYPE OF BITUMINOUS MATERIAL		(1 Place)	 *

(415AREP, 08/18/22)

SECTION 415	ASPHALTIC CONCRETE (ASPHALT-RUBBER) – END PRODUCT:

415-3			Materials:  of the Standard Specifications is modified to add:

For comparative purposes, quantities shown in the bidding schedule have been calculated based on the following data:

| Unit Weight, lb./cu. ft. | XXX |
| --- | --- |
| Asphalt-Rubber, % | X.X |
| Mineral Admixture, % | 1.0 |

415-3.04		Bituminous Material:  the first paragraph of the Standard Specifications is revised to read:

Bituminous material shall be asphalt-rubber conforming to the requirements of Section 1009 of the specifications.  The asphalt-rubber shall be CRA Type XXXXX.  The crumb rubber gradation shall conform to the requirements of Section 1009 of the specifications.

415-9(E)		Acceptability:  of the Standard Specifications is modified to add:

The contractor may submit a written proposal without an engineering analysis for Engineer’s evaluation and approval, that a mixture properties lot with a PT value lower than 50 on gradation and/or asphalt cement content be allowed to remain in place if the application of a Self-Directed Target Change as outlined in Subsection 415-4 of the specifications would bring the PT value to 50 or greater. This written request shall also include the proposed revised PT calculations. Upon Engineer’s verification and approval of the request, the material shall remain in place at the maximum negative pay factor(s) specified in this Subsection for the mixture properties lot in reject.