(608 PANEL, 06/17/21)

SECTION 608	SIGN PANELS:

608-1			Description:  of the Standard Specifications is revised to read:

The work under this section shall consist of furnishing and installing sign panels in accordance with the details shown on the plans and the requirements set forth herein.

The sign panels shall be of the following types:

(A)	Extruded Aluminum Sign Panels with Direct-Applied, or 	Demountable Characters;

(B)	Flat Sheet Aluminum Sign Panels with Direct-Applied, Electronic-Cut, or Screen-Printed Characters;

(C)	Warning, Marker, and Regulatory Sign Panels;

(D)	Route Shields for Installation on Sign Panels; and/or

(E)	EXIT ONLY Panels for Installation on Sign Panels.

608-2.02		Extruded Aluminum Sign Panels with Direct-Applied, Digitally-Imaged, or Demountable Characters:  the title and the third paragraph of the Standard Specifications are revised to read:

608-2.02		Extruded Aluminum Sign Panels with Direct-Applied or Demountable Characters:

The letters, numerals, symbols, borders and other features of the sign message shall be direct-applied, or demountable, and shall conform to the requirements of Subsection 608-2.08 or Subsection 608-2.09 of the specifications.

608-2.03		Flat Sheet Aluminum Sign Panels with Direct-Applied, Digitally Imaged, Electric-Cut or Screen-Printed Characters: the title and the fifth paragraph of the Standard Specifications are revised to read:

608-2.03		Flat Sheet Aluminum Sign Panels with Direct-Applied, Electronic-Cut, or Screen-Printed Characters:

Messages shall be reflectorized white or, if called for on the plans, opaque black, and shall be produced by either screen printing, direct-applying, or electronic cutting, as specified under Subsections 608-2.09 of the specifications.

608-2.05		Route Shields (For Installation on Sign Panels):  The first paragraph of the Standard Specifications is revised to read:

Route shields may be demountable or direct-applied.

608-2.06		EXIT ONLY Panels (For Installation on Sign Panels): the first paragraph of the Standard Specifications is revised to read:

EXIT ONLY panels may be demountable or direct-applied.  Demountable EXIT ONLY panels shall be attached to the sign panel with self-plugging aluminum blind rivets.

608-2.10		Digitally-Imaged Characters: of the Standard Specifications is hereby deleted: