(609DRSFD, 01/21/21)

SECTION 609 	DRILLED SHAFT FOUNDATIONS:

609-1.03		Installation Plan: The third paragraph of the Standard Specifications is revised to read:

Unless otherwise specified in the Special Provisions, foundations of 4 feet or less in diameter and 30 feet or less in length utilized in light pole and sign post foundations shall be exempt from the requirement to submit an installation plan, perform integrity testing, conduct a drilled shaft preconstruction meeting, and construct a confirmation shaft.

609-3.02		Confirmation Shafts: The third paragraph of the Standard Specifications is revised to read:

When shown on the plans or when directed by the Engineer in writing, the reaming of bells or development of rock sockets at specified confirmation shaft holes shall be required to establish feasibility in specific soil strata.