*** USE IN CONJUNCTION WITH THE FOLLOWING PAY ITEMS: 7016075 & 7016079 ***

(701FLGSV, 03/03/22)

SECTION 701	MAINTENANCE AND PROTECTION OF TRAFFIC:

701-3.13		Flagging Services:  of the Standard Specifications is revised to read:

Flagging services shall consist of either contracted labor or law enforcement officers with their vehicles. Authorized law enforcement officers include those who meet the requirements of a Peace Officer under ARS Title 38, generally including anyone certified by the Arizona Peace Officer and Training Board (POST). Vehicles must be authorized emergency vehicles and meet the requirements of ARS Title 28. Any exceptions to either the qualifications of the officer or the type of vehicle must be approved by the State Construction Engineer.  The Engineer will determine the type of flagger needed, and may adjust the relative number of hours of each type of flagger specified in the traffic control plan.

The contractor shall be responsible to procure civilian flaggers or law enforcement officers.  When procuring law enforcement officers, the contractor shall contact the relevant agency at least two days, excluding weekends and holidays, before flagging services will be required.  Such contact must be made between the hours of 7:00 A.M. and 5:00 P.M. (M.S.T.).

In the event that local enforcement officers or DPS officers are temporarily unable to provide flagging services, the contractor shall ensure that traffic control is maintained and all personnel are protected, either by providing civilian flaggers or through other means as approved by the Engineer.  No adjustments to the contract will be allowed for any delays resulting from the unavailability of local enforcement officers or DPS officers.

A law enforcement officer shall not work more than 12 consecutive hours without the permission of the State Construction Engineer unless an emergency situation exists which, in the opinion of the Engineer, requires that the officer remain in the capacity of a flagger.

The contractor shall furnish verification to the Engineer that all civilian flaggers have completed a recognized training and certification program.  Flaggers certified by the American Traffic Safety Services Association (A.T.S.S.A.) or by the National Safety Council shall be acceptable.  Certification through other programs offering flagger training must be approved by the Engineer.  Flagger certification must be current.  Training and certification shall be required at least once every four years.

701-6.07		Pilot Services, and Flagging Services:  of the Standard Specifications is revised to read:

The accepted quantities of pilot vehicles, measured as provided above, will be paid for at the unit bid price for pilot vehicles with driver, which price shall be full compensation for the work, complete in place including, but not limited to, furnishing and maintaining the vehicle and furnishing the driver. Overtime hours for pilot vehicles will be paid for at the unit bid price for pilot vehicle with driver. No additional payment will be made for overtime hours, the cost being considered as included in the unit bid price.

The accepted quantities of flagging services provided by the law enforcement officers and civilian flaggers, measured as provided in Subsection 701-4.04(F) of the specifications, will be paid for at the unit bid price, which price shall be full compensation for the work, complete in place, including all overhead costs and fringe benefits. Overtime hours for law enforcement officers and civilian flaggers will be paid for at the respective unit bid prices. No additional payment will be made for overtime hours, the cost being considered as included in the unit bid price for local enforcement officers and civilian flaggers.

The accepted quantities of flagging services provided by the law enforcement officers, measured as provided above, will be paid for at the contract unit price per hour, which price shall be full compensation for the work complete in place. If needed, travel time may be paid on a case by case basis, as evaluated by the Engineer in accordance with the requirements of Subsection 104.02 of the specifications.