(701EATERM, 08/19/21)

ITEM 7010025 -	TEMPORARY IMPACT ATTENUATION DEVICE (IN-LINE ENERGY ABSORBING TERMINAL):

ITEM 7010026 -	TEMPORARY IMPACT ATTENUATION DEVICE (IN-LINE ENERGY ABSORBING TERMINAL) (IN USE):

Description:

The work under these items shall consist of furnishing, installing, maintaining, and removing in-line energy absorbing terminals in conformance with the details shown on the project plans, and in accordance with the special provisions.

Materials:

In-line energy absorbing terminals shall meet the evaluation criteria for Test Level 3 per MASH (AASHTO Manual for Assessing Safety Hardware) for a speed of 62 miles per hour.

The delineation for the terminals shall conform to the requirements of the Standard Drawing M-34 or in accordance with the manufacturer’s recommendations.  The retroreflective sheeting type shall conform to the requirements of Section 1007 of the specifications.

The contractor shall provide, at the preconstruction conference, a certificate of compliance, conforming to the requirements of Subsection 106.05 of the specifications. The contractor shall include the project number on the submittal. The contractor shall submit the certification at least two weeks prior to the intended use of the terminal.

In regions that are subject to cold weather conditions, if the water filled In-line energy absorbing terminals elements that are prone to freezing are used, appropriate antifreeze agents shall be used in accordance with the manufacturer’s recommendations. The antifreeze materials must be approved by the ADOT Environmental Planning and may require a leak containment as part of the system. If required by the ADOT Environmental Planning, system components containing chemicals must be labeled per OSHA requirements.

Construction Requirements:

The contractor shall install the in-line energy absorbing terminal at the locations shown on the project plans, in accordance with the manufacturer’s instructions and to the satisfaction of the Engineer.

Any traffic control device which will require in-line energy absorbing terminal shall be installed by the contractor only after the Engineer’s approval of the terminal’s use.

At the preconstruction conference meeting, the contractor shall provide in writing, a plan to repair and/or replace any damaged unit during the construction for the Engineer’s approval.

Any unit which has been excessively damaged prior to installation, as determined by the Engineer, shall not be used.

Any unit damaged during or after the installation by the contractor’s operations shall be repaired or replaced with an undamaged unit by the close of that work shift, at no additional cost to the Department. Any unit damaged by the travelling public, the contractor shall replace the unit within 36 hours of identifying the damage.

Method of Measurement:

Temporary Impact Attenuation Device (In-Line Energy Absorbing Terminal) will be measured by the unit for each in-line energy absorbing terminal furnished and installed.

Temporary Impact Attenuation Device (In-Line Energy Absorbing Terminal) (In Use) will be measured by the each-day for each 24-hour day that an in-line energy absorbing terminal is in place and functional for the “in use” condition as specified in Subsection 701-4.01 (B) of the specifications.

Basis of Payment:

The accepted quantities of Temporary Impact Attenuation Device (In-line Energy Absorbing Terminal), measured as provided above, will be paid for at the contract unit price per each, which price shall be full compensation for the work including delineation, maintenance and removal of the terminal, complete in place.

Fifty percent of the contract unit price for the in-line energy absorbing terminal will be paid upon satisfactory installation and fifty percent of the contract unit price will be paid upon final removal of the installed terminal.

Measurement and payment for an in-line energy absorbing terminal which is damaged by the traveling public will be made in accordance with the requirements of Subsection 109.04 of the specifications.

The accepted quantities of Temporary Impact Attenuation Device (In-line Energy Absorbing Terminal) (In Use), measured as provided above, will be paid for at the contract unit price per each-day, which price shall be full compensation for work, as specified herein and as shown on the project plans, complete in place.