(701TPRS, 02/16/23)

ITEM 7017050 -	TEMPORARY PORTABLE RUMBLE STRIPS (INSTALLATION AND REMOVAL):

ITEM 7017051 - 	TEMPORARY PORTABLE RUMBLE STRIPS (IN-USE):

1.0 	Description:

The work under these items shall consist of furnishing, installing, maintaining and removing transverse Temporary Portable Rumble Strips (TPRS) at the locations shown on the project plans and as approved by the Engineer.

2.0 	Materials:

The TPRS unit shall conform to the following requirements. It shall:

- Be black, white or orange in color, but not used in combination.

- Be constructed from a polymer, or other similar, durable material.

- Require no adhesives or anchors for installation.

- Weigh at least 100 lbs.

- Have a profile no greater than ¾ inch.

- Be at least 10 inches wide.

- Require no assembly prior to deployment.

- Withstand surface temperatures of 0 – 180 degrees F without degradation in deployment, use, or safety.

- Have integrated, ergonomic handles on both ends of the TPRS.

- Have grooved designs on the bottom to prevent hydroplaning.

- Have raised designs on the top, with leading and trailing beveled edges to facilitate the safe traversing of motorcycles.

- Have durable orientation indicators to prevent improper deployment.

- Be flexible along the length of the strip to facilitate conformity to the road surface.

- Withstand vehicles with a maximum weight of 80,000 pounds; and retain original placement with minimal movement such that performance is not compromised.

The contractor shall submit to the Engineer, prior to the installation of the TPRS units, a certificate of compliance, conforming to the requirements of Subsection 106.05 of the specifications. The contractor shall include the project number on the submittal.

3.0 	Construction Requirements:

3.01 	General:

TPRS shall be used under the following conditions:

- When the work duration occupies a location for one hour or more.

- When the work zone speed limit does not exceed 45 mph.

- On unmarked two-lane roadways at least 18 feet wide or wider.

In the case of an Emergency Event, the contractor may use TPRS with the approval of the Engineer. An Emergency Event is defined as an incident that requires the ADOT Incident Response Unit (IRU) or Maintenance to respond.

TPRS shall not be used on loose gravel, bleeding asphalt, heavily rutted or unpaved surfaces, or if the work zone is in snowy or icy weather conditions.

TPRS shall be installed in accordance with the manufacturer’s recommendations.

The placement of the TPRS may be based on field conditions as directed by the Engineer.

The TPRS installation shall be non-invasive to prevent any damage in the pavement.

A TPRS Array shall consist of three TPRS units placed perpendicular to the centerline and parallel to one another in accordance with the spacing requirements specified in the plans.

3.02 	Design and Layout:

When the TPRS units are installed, the Rumble Strips advance warning sign(s), W17-2 shall be used as shown on the plans. When traffic queues prior to the TPRS, the TPRS  and Rumble Strips advance warning sign(s) shall be relocated in advance of the queue  to serve as a warning device to road users, as directed by the Engineer.  The TPRS unit shall be placed at the beginning of the operation in accordance with the following guidelines; however, they shall be in place without moving as the work advances,  unless otherwise approved by the Engineer.

- The TPRS unit shall be installed perpendicular to the longitudinal pavement markings.

- The TPRS unit shall be placed across the entire travel lane but not intruding into the opposing travel lane. If the length of the TPRS unit is more than the width of the travel lane, the TPRS unit shall be extended to the shoulder.

- The TPRS unit shall remain in place without moving for the entire work duration and removed upon conclusion of working hours or when workers are no longer present.

- Removal of the TPRS units shall coincide and be in conjunction with the removal of the Rumble Strips advanced warning sign(s).

4.0 	Method of Measurement:

TPRS (Installation and Removal) will be measured as a unit for each complete TPRS array furnished, installed, and removed at the project site.

TPRS (In-Use) will be measured as a unit for each complete TPRS array used by the day for each day that a TPRS array is in place and functional for the "In-Use" condition, regardless of the number of times the unit is realigned or adjusted in place.

5.0 	Basis of Payment:

The accepted quantities of TPRS (Installation and Removal), measured as provided  above, will be paid for at the contract unit price per each TPRS array, which price shall be full compensation for the work, complete in place, including furnishing, installing,  and  removing the TPRS units.

No measurement or payment will be made for the relocation of TPRS units, the cost being considered as included in the cost of the contract item for TPRS (Installation and Removal).

The accepted quantities of TPRS (In-Use), measured as provided above, will be paid for  at the contract unit price of each TPRS array in use per each day, which price shall be full compensation for the work, complete in place, including furnishing all material, equipment and labor for the  maintenance, realignment and adjustment  of the TPRS (In-Use) units.

TPRS units that are lost, stolen, destroyed, or are deemed unacceptable by the Engineer, while in use on a project shall be replaced by the contractor, at no additional cost to the Department.