(928LONG, 09/16/21)

SECTION 928	GROUND-IN RUMBLE STRIP: the title and text of the Standard Specifications are revised to read:

SECTION 928	LONGITUDINAL GROUND-IN RUMBLE STRIP:

928-1			Description:

The work under this item shall include furnishing all tools, equipment and labor necessary to install longitudinal ground-in rumble strips of the size and at the locations specified on the project plans or as directed by the Engineer.

928-2			Blank:

928-3			Construction Requirements:

Longitudinal ground-in rumble strips shall be constructed by mechanically milling or grinding grooves into new or existing asphalt concrete pavements.  The grooves shall be constructed with a cylindrical configuration in the direction of the traffic flow, and shall be placed in accordance with the details shown on the project plans. Exceptions in longitudinal ground-in rumble strips not shown on the plans will be designated by the Engineer.

The grooves shall be constructed with equipment specifically designed to remove such material by means of grinding to a controlled line and grade.  The equipment used shall be capable of removing the existing asphaltic concrete to the dimensions and tolerances specified on the plans.  The removals shall be accomplished in a manner which does not scar the surface of the adjacent pavement.  Grooves shall not be constructed in new asphaltic concrete pavements for a minimum of three days after placement, or 10 days after placement of new asphaltic concrete pavements with asphalt-rubber.  Construction of the grooves may be allowed in less time if the contractor can demonstrate that the pavements are sufficiently cured.

The contractor shall place a continuous control line as a guide for installation.  The milling machine shall also be equipped with an approved guide that is clearly visible to the operator so that proper alignment of the grooves will be obtained.  Lateral deviation of the milled rumble strips shall not exceed one inch in any 100 foot interval.

The contractor shall also use high pressure air to remove any millings and debris remaining after the use of the equipment's vacuum recovery system. Such debris shall be removed to the shoulder and disposed of in a manner approved by the engineer.

928-4			Method of Measurement:

Longitudinal ground-in rumble strips will be measured by linear foot at the locations shown in the project plans and in accordance with the specifications.

Gaps in the pattern and exceptions in the longitudinal ground-in rumble strips will not be included in the measurement.

No measurement will be made for the removal and disposal of milling and debris.

928-5			Basis of Payment:

The accepted quantities of longitudinal ground-in rumble strips, measured as provided above, will be paid for at the contract unit price per linear foot, which price shall be full compensation for the work complete in place as shown on the project plans and as specified herein.

No additional payment will be made for the removal and disposal of milling and debris, the cost being considered as included in the price of the contract item.